Check Yourself

Activity 1

See the diagram on page 9 of the unit.

Activity 2

An example from each situation mentioned:

| Machine with Hydraulic System | Purpose of the Hydraulic System | Output Movement Needed | Actuator/s Used |
| --- | --- | --- | --- |
| Surface Grinder | Move machine table to and fro under grinding wheel. | Linear | Cylinder |
| Bulldozer | Lift and lower scraper blade. Drive tracks. | Semi-rotary* Rotation | Cylinder Motor |
| Excavator (JCB) | Operate arm. Lift off wheels. | Semi-rotary* Rotation | Cylinders Cylinders |
| Undercarriage Aircraft | Lower and raise wheels and support aircraft. | Semi-rotary* | Cylinders |
| Tractor Farm | Lift and lower implements. Drive wheels and power take offs. | Semi-rotary* Rotation | Cylinder Motors |
| Stabilisers Ships | Extend and adjust to stop ship rolling in heavy seas. | Linear Semi-rotary* | Cylinders |
| Hydraulic Tailboard Lorry | Lift loads to level of lorry platform. | Linear | Cylinders |

You will probably have found many more in the locations or books accessible to you.

* Semi-rotary means part of a rotation

Activity 4

N/m2, Pascals, bar, kg/cm2, lbs/inch2, atmospheres.

Activity 5

Pascal’s Law States:

Pressure applied to a confined liquid is transmitted with equal intensity in all directions within the system and will exert an equal force on equal areas. See diagram below.

Activity 6

Force

Area

1 	a 	Pressure  =

Putting in numbers from the drawing

60

100

Pressure  = 		= 	0.61N/mm 2

Force  =  Pressure  x  Area

Putting in area of brake cylinder and our answer from above

Force  =  0.6  x  800  =  480 N

or

FIN  x  AL

AS

FOUT  =

Put in appropriate values from drawing

60  x  800

1000

FOUT  = 			=	 480 N

b 	The pressure in the liquid is acting against the much larger area of 	the brake cylinder’s piston. As its area is 8 times larger and the 	‘pressure acts on equal areas with equal force’ then output force 	must be 8 times bigger.

2 	Pressure 	=

so in this case

Force

Area

Pressure  =  600  =  0.75N/mm2

Force  =  Pressure  x  Area Force  =  0.75  x  16000  =  12000N

As the cylinder is not rising, or falling, but just balancing the load this must be its weight.

3 	The input force is 600N

The output force from Question 2 is 12000.

12000

600

The output force is 			times bigger which is 20 times.

This is the FORCE RATIO of the hydraulic lever.

You could also have found the FORCE RATIO without knowing the forces involved as long as you know the area of both cylinder’s pistons.

Simply divide as shown

Area of Large Cylinder

Area of Small Cylinder

Force Ratio 	=

Try it for the hydraulic cylinders in Question two. You should get the same answer: 20.

Activity 7

1 	Machine tool table feeds.

2 	Hydraulic tailboards, lifts and hoists.

3 	Hydraulic robot arms.

4 	Extrusion rams

Activity 8

Reservoir, filter, relief valve, control valve, throttle valve and the pump.

Activity 9

a 	The fluid in the bottom of the ram is TRAPPED and as enclosed 	liquids are INCOMPRESSABLE, the ram cannot descend until the 	valve is opened to let the fluid out.

If you are not convinced about the incompressibility of enclosed liquids; try this experiment:

1 	Get a tin of your favourite drink

2 	Try to squash it in your hand

3 	Drink the contents

4 	Try and squash the empty tin

The tin is so weak, without the support of the incompressible confined liquid inside it, it collapses easily.

b 	See 'How the system works'

Activity 10

1	Block Diagram of the Plastic Moulding Machine Circuit

2 	The first part includes the reservoir, filter, pump and relief valve. 	These units are common to most hydraulic systems and together 	are often called the ‘power pack’.

3 	The ram has only one connection to its control valve. Fluid from the 	control valve is pumped in through it, to raise the ram and is 	allowed to exhaust out through it when the ram is lowered by 	gravity. Such cylinders are called SINGLE ACTING.

The cylinder is driven in both directions by the fluid, so it must have 	two connections to its control valve. This is called a DOUBLE 	ACTING CYLINDER.

4 	The control valve for the car lift has three connection points called 	ports.

The control valve for the cylinder has four connection ports.

The car lift control valve is manually controlled by a lever.

The moulding machine valve is operated by an electrical signal and 	when this signal stops the valve is returned to its other position by a 	spring.

Activity 11

The incompressible fluid trapped at one end of the cylinder would not allow its piston to move.

| Hydraulic Component | BS 2917 Symbol |
| --- | --- |
| 4/2 Valve Electrical (solenoid (control) Spring Return |  |
| Reservoir |  |
| Filter |  |
| Pump (Fixed Capacity) |  |
| Throttle Valve |  |
| Double Acting Cylinder with Single Piston Rod |  |
| Pressure Relief Valve |  |

Activity 13

a 	Double acting cylinder

b 	(Four two) 4/2 lever operated, detent valve.

c 	Adjustable throttle valve

d 	Pressure relief valve

e 	Pump

f 	Filter

g 	Reservoir

Activity 14

See text following drawing.

Activity 15

The main points your description should include are:

To drive the winch clockwise solenoid A is energized.

The valve is moved so port P is connected to A and port B is 	connected to T.

To drive the winch anti-clockwise solenoid B is energised.

The valve then moves so that port P is connected to B and port A is 	connected to T.

When either solenoid is de-energised the springs return the valve to 	its central neutral position.

Fluid is trapped in the motor to stop and hold it in position.

The pumps output is returned to the reservoir by the relief valve.

The throttle valve controls the speed of the winch.

The return line stops the load running away (overhauling) when it is 	being lowered.

The functions of the relief valve, pump, filter and reservoir are as in 	other power packs we have described.

Activity 16

To be Tutor marked.